| № | Term/Abbreviation | Definition | Example Usage in the Article |
| --- | --- | --- | --- |
| 1 | Comb-drive actuator (Гребенчатый привод) | A microelectromechanical actuator using interdigitated fingers to generate electrostatic force. | "Two comb-drive actuators are used to initiate normal and frictional forces." |
| 2 | Static friction coefficient (Коэффициент статического трения) | The ratio of maximum static friction force to normal force before motion begins. | "Static friction coefficient decreases with increasing normal force." |
| 3 | Dynamic friction coefficient (Коэффициент динамического трения) | The ratio of kinetic friction force to normal force during motion. | "Dynamic friction coefficients were measured at 0.31–0.33." |
| 4 | Single crystal silicon (Монокристаллический кремний) | A pure form of silicon with a continuous crystal lattice, used in MEMS. | "The tester is fabricated from single crystal silicon." |
| 5 | Wear debris (Продукты износа) | Particles generated due to friction and wear between surfaces. | "Wear debris forms agglomerates due to adhesion forces." |
| 6 | Elastic deformation (Упругая деформация) | Temporary shape change under stress, reversible upon load removal. | "Silicon exhibits elastic deformation under normal force." |
| 7 | ICP etching (Плазменное травление ICP) | A microfabrication technique using inductively coupled plasma to remove material. | "The device is patterned via ICP etching." |
| 8 | Adhesion force (Сила адгезии) | Attractive force between two surfaces in contact. | "Debris agglomerates due to strong adhesion forces." |
| 9 | Normal force (Нормальная сила) | Perpendicular force exerted between contacting surfaces. | "Normal force is applied via a comb-drive actuator." |
| 10 | MEMS (Микроэлектромеханические системы) | Miniaturized devices combining mechanical and electrical components. | "The tribotester simulates MEMS side-contact conditions." |
| 11 | Bulk micromachining (Объемная микрообработка) | A process to create 3D structures by etching silicon substrates. | "The device is fabricated by bulk micromachining." |
| 12 | Stiffness (Жесткость) | Resistance of a structure to deformation under applied force. | "System stiffness (kₓ) is calculated as 89 µN/µm." |
| 13 | Electrostatic force (Электростатическая сила) | Force generated by electric charges on comb-drive fingers. | "Electrostatic force is calculated using Eq. (1)." |
| 14 | Wear resistance (Износостойкость) | Material's ability to withstand surface degradation. | "Wear resistance is critical for MEMS longevity." |
| 15 | Humidity (Влажность) | Amount of water vapor in the environment. | "Tests were conducted at 21% humidity." |
| 16 | Oxidation (Окисление) | Chemical reaction with oxygen, forming oxides. | "Oxidation increases oxygen content to 21%." |
| 17 | SEM (Сканирующая электронная микроскопия) | Imaging technique for surface morphology analysis. | "Debris is analyzed via SEM." |
| 18 | KOH etching (Травление KOH) | Wet etching using potassium hydroxide for silicon micromachining. | "Shallow grooves are created by KOH etching." |
| 19 | Parasitic capacitance (Паразитная емкость) | Unintended capacitance in circuits affecting performance. | "Guards minimize parasitic capacitance effects." |
| 20 | Tribologyl (Трибология) | Study of friction, wear, and lubrication. | "The work advances microscale tribology." |